GTM965500P

Test Report

24V Model

GlobTek Engineering

Generated: April 16, 2026

# Test Summary

Project: GTM965500P  |  24V Model  |  Total Test Items: 22

| Status | OK / Resolved | For Review | Regression Req'd | In Progress | Has Issue | Invalid/Incomplete Test | Unknown |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Count | 2 | 10 | 0 | 1 | 0 | 0 | 9 |

## 24V

| Test Item | Status | Link |
| --- | --- | --- |
| Input |  |  |
| No Load Input Power | Marked for review | View → |
| Input Voltage | OK / Resolved | View → |
| Main Output |  |  |
| Turn-On Delay | Marked for review | View → |
| Load Regulation | Marked for review | View → |
| Voltage Ripple | OK / Resolved | View → |
| Transient Response | Marked for review | View → |
| Startup Overshoot | Marked for review | View → |
| Hold-Up Time | Marked for review | View → |
| Remote-Sense (SNS) | Marked for review | View → |
| Output Voltage Fine Adjustment | Marked for review | View → |
| Protections |  |  |
| Over-Power Protection (OPP) | Marked for review | View → |
| Short-Corcuit Protection (SCP) | Marked for review | View → |

# Changes Required

2 changes flagged across all tests.

| Voltage | Test | Change Description |
| --- | --- | --- |
| 24V | Over-Power Protection (OPP) · Vision314 · 2026-04-01 | COMPONENT ¦ OLD VALUE ¦ NEW VALUE -- ¦ -- ¦ -- C26 ¦ 6.8nF ¦ 5.6nF C27 ¦ 3.3nF ¦ 2.2nF |
| 24V | Output Voltage Fine Adjustment · Vision314 · 2026-04-02 | Req of Ra8, Ra9 and Ra11, Ra12 is 90.7kOhm Ra8, Ra11 = 88.7kOhm Ra9, Ra12 = 2kOhm Ra14 = 16kOhm Output range: 22.8V -> 25.2V |

# Tests Not Yet Run

10 tests have not yet been run.

## 24V

| Test Item | Status | Link |
| --- | --- | --- |
| Protections |  |  |
| Over-Voltage Protection (OVP) | In Progress | View → |
| Environmental / Reliability |  |  |
| Component Thermal Stress | Unknown | View → |
| Component Electrical Stress | Unknown | View → |
| Safety |  |  |
| Dielectric Withstand (Hi-Pot) | Unknown | View → |
| Output Touch Current | Unknown | View → |
| Earth Leakage Current | Unknown | View → |
| EMC |  |  |
| Conducted Emissions | Unknown | View → |
| Radiated Emissions | Unknown | View → |
| Electrostatic Discharge (ESD) | Unknown | View → |
| Electrical Fast Transient (EFT) | Unknown | View → |

# Detailed Test Results

## 24V

| 24V | No Load Input Power | OPEN | Marked for review |
| --- | --- | --- | --- |

Specification / Procedure

Spec: < 500mW @ 240Vac (main-output disabled, i.e. remote-off low)

Solder RO pin to AUX GND to disable output without additional regulation board load

Use Yokogawa WT3000 with 20 second acquisition time (update rate)

Results

Vision314 · 2026-04-15 [LATEST] 570mW

Comments (1)

lauxtim · 2026-04-15 [LATEST] See: https://github.com/GlobTek-Engineering/GTM965500P/issues/24#issuecomment-4255997812

| 24V | Turn-On Delay | OPEN | Marked for review |
| --- | --- | --- | --- |

Specification / Procedure

Spec: < 2 seconds @ 115Vac

Measure AC input voltage with differential probe, passive probe on DC output. Check the time between AC turn-on and DC turn-on, at full load.

Results

Vision314 · 2026-04-06 239ms @ 22.9A Vision314 · 2026-04-16 [LATEST] GTSZ Condition; Spec (mS); Reading (mS); Picture; Result. 100Vac, 22.916A; 2000; 1150; PASS. 240Vac, 22.916A; 2000; 710; PASS.

| 24V | Load Regulation | OPEN | Marked for review |
| --- | --- | --- | --- |

Specification / Procedure

Spec: ± 1.0% (measured at output connector)

Check the stabilized DC output voltage at no-load (0%). Check the stabilized DC output voltage at full load (100%).

% Load regulation = [ V(no_load) - V(full_load) / V(full_load) ] * 100%

Measure at 90Vac

Measure at 264Vac

Results

Vision314 · 2026-04-06 Input; V (no load); V (full load); Load Regulation (%). 90Vac; 24.036; 24.033; 0.01%. 264Vac; 24.036; 24.032; 0.02%. Vision314 · 2026-04-16 [LATEST] GTSZ Input Voltage; 12V; Load (A); Test Result. 90; 24.094; 0; PASS. 24.02; 5.73; PASS. 24.02; 11.46; PASS. 24.01; 17.19; PASS. 24.01; 22.916; PASS. 115; 24.094; 0; PASS. 24.02; 5.73; PASS. 24.02; 11.46; PASS. 24.01; 17.19; PASS. 24.01; 22.916; PASS. 230; 24.094; 0; PASS. 24.02; 5.73; PASS. 24.02; 11.46; PASS. 24.01; 17.19; PASS. 24.01; 22.916; PASS. 264; 24.094; 0; PASS. 24.02; 5.73; PASS. 24.02; 11.46; PASS. 24.01; 17.19; PASS. 24.01; 22.916; PASS.

| 24V | Voltage Ripple | CLOSED | OK / Resolved |
| --- | --- | --- | --- |

Specification / Procedure

Spec: < 1% pk-pk (measured with a 47µF low-ESR cap + 0.1µF ceramic cap, measured @ 20MHz BW)

Prior to recording data, check whether 90Vac or 264Vac yields worse results, if at all. Record data using the worst case input voltage.

Using the ripple probe fixture and matching 1x probe, connect close to the output connectors on the PCB, measure ripple with oscilloscope. Ripple is inclusive of HF (100KHz) and LF (120Hz) components. Timebase should capture 120Hz component (e.g. 2-5ms/div)

Three conditions, adjust potentiometer accordingly:

(95% * V_out)

(100% * V_out)

(105% * V_out)

Results

Vision314 · 2026-04-06 [LATEST] ALL PASS 1% ripple = 240mV Output Voltage; Picture; Measured Ripple. 95% Vout; 29.2mV. 100% Vout; 51.7mV. 105% Vout; 84.2mV.

| 24V | Transient Response | OPEN | Marked for review |
| --- | --- | --- | --- |

Specification / Procedure

Spec: ± 5% deviation max. (with 50% load step), recovery to within ±1% within 1ms

Taken at nominal input voltage (e.g. 115Vac)

Measure with ripple probe fixture PCB, Measure at power supply PCB. Measured with AC coupling.

Step load + load dump:

0 -> 25%

25% -> 50%

50 -> 75%

75 -> 100%

8 measurements total

Results

Vision314 · 2026-04-06 [LATEST] Load Step Transition; Deviation; Recovery Time. 0A → 5.7A; 493mV; <1ms. 5.7A → 11.5A; 248mV; <1ms. 11.5A → 17.2A; 255mV; <1ms. 17.2A → 22.9A; 303mV; <1ms. Load Dump Transition; Deviation; Recovery Time. 0A → 5.7A; 220mv; <1ms. 5.7A → 11.5A; 182mV; <1ms. 11.5A → 17.2A; 181.7mV; <1ms. 17.2A → 22.9A; 207.5mV; <1ms. 1% = 240mV Load Step Load Step; Picture; Deviation; Recovery Time. 0A → 5.7A; 493mV; <1ms. 5.7A → 11.5A; 248mV; <1ms. 11.5A → 17.2A; 255mV; <1ms. 17.2A → 22.9A; 303mV; <1ms. Load Dump Load Dump; Picture; Deviation; Recovery Time. 0A → 5.7A; 220mv; <1ms. 5.7A → 11.5A; 182mV; <1ms. 11.5A → 17.2A; 181.7mV; <1ms. 17.2A → 22.9A; 207.5mV; <1ms.

Comments (1)

lauxtim · 2026-04-14 [LATEST] In reviewing the transient response for 12, 24V, and 54V, many times the peak deviation does not ever exceed the 1% recovery threshold, so the recovery time should probably be marked as "N/A" or "Dev. < 1%" for those items. (Cannot quantify how long it takes to recovery if it never deviated outside of our bounds to start with.) Above, all of the load steps exceed 1% deviation (240mV), so then we need to assess whether the peak deviation has exceeded the 5% max or not. From inspection, seems like most are in the 200-300us range. Writing <1ms is OK as a pass/fail. For the future, you *could* record the actual recovery time. No need to re-test, just update those which never deviated outside of 1% to "Dev. < 1%" or similar. As a note: I'd probably set the cursors as follows: Y(1): At peak of measured waveform (measured max deviation) Y(2): At the 1% recovery threshold X(1): At t=0 (the moment the load step is applied) X(2): At the moment the voltage recovers to less than Y(2), the recovery threshold (measured recovery time) It may make sense (in general, all types of tests), to present data with the specified limits presented in line. This way, you can easily see (at a glance) whether the spec is met or not, no need to scroll up or remember what the spec is. Load Dump Condition; Picture; Max Deviation; Max Deviation (Spec); Recovery Time; Recovery Time (Spec).

| 24V | Startup Overshoot | OPEN | Marked for review |
| --- | --- | --- | --- |

Specification / Procedure

Spec: <5%

Measure with oscilloscope in DC coupling.

Measure at 90Vac and 264Vac, at:

No load

Full load

4 measurements total

Results

Vision314 · 2026-04-06 [LATEST] Output Voltage Overshoot Condition; 90 Vac; 264 Vac. No Load; 0mV; 0mV. Full Load; 0mV; 0 mV. Output Voltage Overshoot Condition; 90 Vac; 264 Vac. No Load. Full Load.

| 24V | Hold-Up Time | OPEN | Marked for review |
| --- | --- | --- | --- |

Specification / Procedure

Spec: 20ms typ. (100% load), 20ms min. (95% load), 50ms typ. (50% load)

Attach differential probe to AC input. Attach passive probe to output. Set load according to the above. Measure the time between AC turn-off and DC output falling to approx. 95% * nominal V_out.

Results

Vision314 · 2026-04-02 [LATEST] Input Voltage; Output Voltage; Hold Up Time (ms) 100% Load (22.9 A); Hold Up Time (ms) 50% Load (11.5 A). 90Vac; 22.8; 28.3; 56.3. 24; 27.5; 52.3. 25.7; 25.5; 50.7. 264Vac; 22.8; 28.9; 56.3. 24; 27.3; 52.3. 25.7; 25.7; 50.1.

| 24V | Remote-Sense (SNS) | OPEN | Marked for review |
| --- | --- | --- | --- |

Specification / Procedure

Spec: Up to 500mV total cable-drop compensation, reverse and short circuit protected. Connect SNS+/SNS- leads to point-of-load with twisted pair wire. (Note 1)

Attach sufficiently long wires between the output and the load, which causes a voltage drop exceeding 250mV per wire @ 85% load current. (Rationale for 85%: When remote sense is used, output voltage and output power goes up, when tested with a constant current load. We do not specify operation at P_out>100% rating.)

For instance, the DCR of 1.5m of 12AWG wire is 0.0075ohm. For the 12V model, this yields a drop of 292mV per wire, or 584mV total. Different cable lengths shall be used for other voltage models.

Part A:

- Run at nominal line voltage (e.g. 115Vac)

- Increase the set-point voltage to V_nom + 5% using the potentiometer.

1. Leave the sense leads disconnected from the load. Measure the voltage at the power supply PCB @ 85% load.

2. Connect the twisted-pair sense leads at the point-of-load (at the E-load terminals).

3. Measure the voltage at the power supply PCB again @ 85% load.

The voltage at the PCB should have increased by at least 500mV.

Part B:

1. Detach the SNS leads, reverse the leads (connect SNS+ to -load terminal and SNS- to +load terminal). Check that there is no damage during and after.

Part C:

1. Detach the SNS leads and connect them to each other (short SNS+ to SNS-, alone). Check that there is no damage during and after.

Results

Vision314 · 2026-04-13 [LATEST] Previous issues were caused by broken/missing/incorrect TVS diodes for ZD6 and ZD7. Must be 54V zener ("RE" marking) for all voltage models. 85% load = 19.5A Part A Step #; Voltage @ Supply; Voltage @ Load. 1; 25.2 V; 24.25 V. 3; 25.8 V; 24.9 V. Part B - No damage Voltage @ Supply; Voltage @ Load. 26.45 V; 25.5 V. Part C - No damage Voltage @ Supply; Voltage @ Load. 26.3 V; 25.5 V.

Comments (3)

Vision314 · 2026-04-07 Part A 12AWG; ~6ft; Rwire = 0.013 Ohm. 85% load = 19.5A 1. 25.3 V 2. .. 3. 25.67 V Part B 1. When I connected the SNS- to +load, there was a spark between those two terminals and the supply OVP'd. On startup the output voltage goes to ~40V and OVP's. Part C 1. ... Vision314 · 2026-04-08 The ZD7 TVS diode was removed previously for other rework troubleshooting When the SNS+ connection touched the negative output terminal, this create some large voltage spike that damaged RJ2 RJ2 Failure Mode: open Replacing RJ2 with another 2.2Ohm resistor fixed the supply Solution: don't remove TVS diodes, we need them... I will finish SNS testing shortly lauxtim · 2026-04-15 [LATEST] See: https://github.com/GlobTek-Engineering/GTM965500P/issues/55#issuecomment-4256097729

| 24V | Output Voltage Fine Adjustment | OPEN | Marked for review |
| --- | --- | --- | --- |

Specification / Procedure

Spec: Via potentiometer, ±5% adjustment range (Note 1)

22.8V -> 25.2V

Run at nominal line voltage (e.g. 115Vac)

Sense leads (SNS) disconnected.

Adjust the potentiometer to minimum and maximum positions. Record the minimum and maximum output voltage, measured at the PCB.

Changes Required

Vision314 · 2026-04-02 [LATEST] Req of Ra8, Ra9 and Ra11, Ra12 is 90.7kOhm Ra8, Ra11 = 88.7kOhm Ra9, Ra12 = 2kOhm Ra14 = 16kOhm Output range: 22.8V -> 25.2V

| 24V | Over-Power Protection (OPP) | CLOSED | Marked for review |
| --- | --- | --- | --- |

Specification / Procedure

Spec: 105-150%, non-latching with auto-recovery

Spec update: Change from "Over-Current Protection" to "Over-Power Protection"

Run at 90Vac and 264Vac.

The specified range applies only for V_out = V_out(nom). (e.g. "12V" @ 12.00V, "24V" @ 24.00V, etc.), voltage measured at the PCB. The nominal design point should be 130 - 135%.

Reference OPP data should also be collected at V_out(-5%) and V_out(+5% + 500mV), for reference although not specified. At V_out(+5% + 500mV), at least 100% output power should be obtainable.

FRANKIE TESTING NOTES:

4/1/2026

Noting current values for regulation board. In order to test Vout max and min (22.8 V -> 25.7V) the following regulation board components were used and give a actual output voltage range of 21.4 V -> 26.5V

Ra8, Ra11 = 84.5kOhm

Ra9, Ra12 = 100Ohm

Ra10, Ra13 = 10kOhm

Ra14 = 7.5kOhm

Changes Required

Vision314 · 2026-04-01 [LATEST] COMPONENT; OLD VALUE; NEW VALUE. C26; 6.8nF; 5.6nF. C27; 3.3nF; 2.2nF.

Results

Vision314 · 2026-04-14 [LATEST] Input Voltage; Output Voltage; OPP Point (A); OPP Point (%); PG Time (ms). 90Vac; 22.8; 33; 144%; 10. 24; 32; 140%; 10. 25.7; 24; 112%; 10. 264Vac; 22.8; 38; 166%; 10. 24; 32; 140%; 10. 25.7; 24; 112%; 10.

Comments (1)

Vision314 · 2026-04-01 [LATEST] Trying the same values as the 12V: COMPONENT; OLD VALUE; NEW VALUE. C26; 6.8nF; 6.8nF. C27; 3.3nF; 2.2nF. Input Voltage; Output Voltage; OPP Point (A); OPP Point (%); PG Time (ms). 90Vac; 22.8; 34; 148%; 0. 24; 32; 140%; 9.5. 25.7; 27; 126%; 9.5. 264Vac; 22.8; 43; 188%; 9.5. 24; 36; 157%; 9.5. 25.7; 27; 126%; 9.5. FRANKIE TESTING NOTES: 4/1/26 Noting current values for regulation board. For testing between -5% to +(5% + 500mV) => 22.8 V -> 25.7V 21.4 V -> 26.5V Ra8, Ra11 = 84.5kOhm Ra9, Ra12 = 100Ohm Ra10, Ra13 = 10kOhm Ra14 = 7.5kOhm

| 24V | Short-Corcuit Protection (SCP) | OPEN | Marked for review |
| --- | --- | --- | --- |

Results

Vision314 · 2026-04-15 [LATEST] PASS

| 24V | Input Voltage | CLOSED | OK / Resolved |
| --- | --- | --- | --- |

Specification / Procedure

Spec: 90 - 264Vac (Nameplate: 100 - 240Vac)

Confirm that full rated power can be drawn over the entire rated input voltage range. Test full load @ 90Vac, 115Vac, 240Vac, 264Vac

Results

Vision314 · 2026-04-16 [LATEST] GTSZ Input Voltage; Output Voltage; Load (A); Test Result. 90; 24.094; 0; PASS. 90; 24.01; 22.916; PASS. 120; 24.094; 0; PASS. 120; 24.01; 22.916; PASS. 240; 24.094; 0; PASS. 240; 24.01; 22.916; PASS. 264; 24.094; 0; PASS. 264; 24.01; 22.916; PASS.

Comments (1)

Vision314 · 2026-04-02 [LATEST] https://github.com/GlobTek-Engineering/GTM965500P/issues/68#issuecomment-4172278539

# Untested — Specifications

## 24V

| 24V | Over-Voltage Protection (OVP) | OPEN | In Progress |
| --- | --- | --- | --- |

| 24V | Component Thermal Stress | OPEN | Unknown |
| --- | --- | --- | --- |

| 24V | Dielectric Withstand (Hi-Pot) | OPEN | Unknown |
| --- | --- | --- | --- |

| 24V | Output Touch Current | OPEN | Unknown |
| --- | --- | --- | --- |

| 24V | Earth Leakage Current | OPEN | Unknown |
| --- | --- | --- | --- |

| 24V | Conducted Emissions | OPEN | Unknown |
| --- | --- | --- | --- |

| 24V | Radiated Emissions | OPEN | Unknown |
| --- | --- | --- | --- |

| 24V | Electrostatic Discharge (ESD) | OPEN | Unknown |
| --- | --- | --- | --- |

| 24V | Electrical Fast Transient (EFT) | OPEN | Unknown |
| --- | --- | --- | --- |

| 24V | Component Electrical Stress | OPEN | Unknown |
| --- | --- | --- | --- |