GTM965500P

Test Report

Model Level Model

GlobTek Engineering

Generated: April 16, 2026

# Test Summary

Project: GTM965500P  |  Model Level Model  |  Total Test Items: 11

| Status | OK / Resolved | For Review | Regression Req'd | In Progress | Has Issue | Invalid/Incomplete Test | Unknown |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Count | 1 | 3 | 0 | 0 | 1 | 0 | 6 |

## Model Level

| Test Item | Status | Link |
| --- | --- | --- |
| Input |  |  |
| Input Current | Marked for review | View → |
| Inrush Current | Marked for review | View → |
| Power Factor | Marked for review | View → |
| Main Output |  |  |
| Power Good (PG) | OK / Resolved | View → |

# Tests Not Yet Run

7 tests have not yet been run.

## Model Level

| Test Item | Status | Link |
| --- | --- | --- |
| Main Output |  |  |
| Remote-Off (RO) | Unknown | View → |
| Standby Output |  |  |
| Output Voltage Regulation | Unknown | View → |
| Fan Output |  |  |
| Output Voltage Regulation | Unknown | View → |
| Protections |  |  |
| Under-Voltage Protection (UVP) | Unknown | View → |
| Over-Temperature Protection (OTP) | Unknown | View → |
| EMC |  |  |
| Lightning Surge | Unknown | View → |
| PFC |  |  |
| PFC Voltage Drop at 90Vac - overload | Has Issue | View → |

# Detailed Test Results

## Model Level

| Model Level | Input Current | OPEN | Marked for review |
| --- | --- | --- | --- |

Specification / Procedure

Spec: 6.5A max. @ 100Vac, 5.2A typ. @ 115Vac, 2.5A typ. @ 240Vac (full load)

Suggest using the 12V unit since it has the lowest efficiency and will have the highest input current.

Results

Vision314 · 2026-04-16 [LATEST] GTSZ 12V Spec (A); Reading (A); Result; Picture. 6.5; 6.128; PASS. 24V Spec (A); Reading (A); Result; Picture. 6.5; 6.056; PASS.

| Model Level | Inrush Current | OPEN | Marked for review |
| --- | --- | --- | --- |

Specification / Procedure

Spec: < 35A @ 115Vac , < 70A @ 240Vac (cold or warm start)

Use Chroma AC source, set start phase angle to 90°. Verify and record AC turn-on waveform, showing 90° turn-on with power supply detached. Connect power supply, Using oscilloscope current probe, measure and record peak value of inrush current.

Inrush current should not depend on output voltage so any voltage can be used.

Results

Vision314 · 2026-04-16 [LATEST] [GTSZ] 12V Voltage; Spec (A); Reading (A); Result; Picture. 115Vac; 35; 14.4A; PASS. 240Vac; 70; 44A; PASS. 24V Voltage; Spec (A); Reading (A); Result; Picture. 115Vac; 35; 18A; PASS. 240Vac; 70; 38.4A; PASS.

| Model Level | Power Factor | OPEN | Marked for review |
| --- | --- | --- | --- |

Specification / Procedure

Spec: > 0.98 @ 115Vac, > 0.97 @ 240Vac typ. (25 - 100% load)

Using Yokogawa WT3000, measure power factor at 25%, 50%, 75% and 100% load, at 115Vac and 240Vac.

Power factor does not depend on output voltage so any voltage can be used.

Results

Vision314 · 2026-04-16 [LATEST] GTSZ 24V, 115Vac, Average Efficiency = 92.102% Parameter; 100%; 75%; 50%; 25%. Percent of Rated Load; 100%; 75%; 50%; 25%. True Power Factor (Watts/VA); 0.9992; 0.9990; 0.9987; 0.9975. Efficiency; 91.763%; 92.566%; 92.886%; 91.193%. 24V, 230Vac, Average Efficiency = 94.247% Parameter; 100%; 75%; 50%; 25%. Percent of Rated Load; 100%; 75%; 50%; 25%. True Power Factor (Watts/VA); 0.9987; 0.9967; 0.9920; 0.9819. Efficiency; 94.652%; 94.625%; 94.415%; 93.295%. 12V, 115Vac, Average Efficiency = 91.907% Parameter; 100%; 75%; 50%; 25%. Percent of Rated Load; 100%; 75%; 50%; 25%. True Power Factor (Watts/VA); 0.9985; 0.9989; 0.9988; 0.9974. Efficiency; 90.751%; 91.875%; 92.853%; 92.151%. 12V, 230Vac, Average Efficiency = 93.553% Parameter; 100%; 75%; 50%; 25%. Percent of Rated Load; 100%; 75%; 50%; 25%. True Power Factor (Watts/VA); 0.9985; 0.9960; 0.9907; 0.9760. Efficiency; 93.438%; 93.786%; 94.010%; 92.976%.

| Model Level | Power Good (PG) | CLOSED | OK / Resolved |
| --- | --- | --- | --- |

Specification / Procedure

Spec: Signal typically goes low ~10ms ahead of AC power failure or entering of protection state

5V logic level, VH = 4.90V typ. @ IPG= 100µA

Select any model output voltage. PG behavior should not depend output voltage.

Select a nominal input voltage (e.g. 115Vac)

At OPP: Step the load above the OPP threshold. Measure the time between PG asserting low and the output voltage going low.

At AC power failure: At 75% load, switch off AC input power. Measure the time between PG asserting low and the output voltage going low.

Results

Vision314 · 2026-04-14 [LATEST] YELLOW - PG Signal PINK - Output Voltage 12V Failure Mode; PICTURE; PG Time. OPP; 9.5ms. AC Power Failure; 11.8ms. OPP Load = 66A (144%) 75% load = 34.4 A 24V Failure Mode; PICTURE; PG Time. OPP; 9.7ms. AC Power Failure; 12.2ms. OPP Load = 32A (139%) 75% load = 17.2 A 54V Failure Mode; PICTURE; PG Time. OPP; 9.7ms. AC Power Failure; 12.4ms. OPP Load = 13A (128%) 75% load = 7.6 A

# Untested — Specifications

## Model Level

| Model Level | Remote-Off (RO) | OPEN | Unknown |
| --- | --- | --- | --- |

Specification / Procedure

Spec: Pull 'Remote Off' to COM to disable main output; leave floating or connect to +5VSB otherwise, IRO = 2mA typ. during disabled/off state

Select a nominal input voltage (e.g. 115Vac).

Connect signal breakout board with RO switch. Capture RO signal going low at switch off. Capture V_out going low in response.

| Model Level | Output Voltage Regulation | OPEN | Unknown |
| --- | --- | --- | --- |

Specification / Procedure

Spec: ± 5.0% (set point tolerance + load/line/cross regulation)

At nominal line voltage (e.g. 115Vac), set main-output load to 50% load, measure the standby output voltage:

1. Fan output = 0% / Standby output: 0%

2. Fan output = 0% / Standby output: 100%

1. Fan output = 100% / Standby output: 0%

2. Fan output = 100% / Standby output: 100%

Then, disable main output via Remote Off (RO) switch, change input voltage to 90Vac, and record the standby output voltage:

1. Fan output = 0% / Standby output: 0%

2. Fan output = 0% / Standby output: 100%

1. Fan output = 100% / Standby output: 0%

2. Fan output = 100% / Standby output: 100%

With Remote Off (RO) still enabled (main output off), change input voltage to 264Vac, and record the standby output voltage:

1. Fan output = 0% / Standby output: 0%

2. Fan output = 0% / Standby output: 100%

1. Fan output = 100% / Standby output: 0%

2. Fan output = 100% / Standby output: 100%

| Model Level | Output Voltage Regulation | OPEN | Unknown |
| --- | --- | --- | --- |

Specification / Procedure

At nominal line voltage (e.g. 115Vac), set main-output load to 50% load, measure the fan output voltage:

Fan output = 0% / Standby output: 0%

Fan output = 0% / Standby output: 100%

Fan output = 100% / Standby output: 0%

Fan output = 100% / Standby output: 100%

Then, disable main output via Remote Off (RO) switch, change input voltage to 90Vac, and record the fan output voltage:

Fan output = 0% / Standby output: 0%

Fan output = 0% / Standby output: 100%

Fan output = 100% / Standby output: 0%

Fan output = 100% / Standby output: 100%

With Remote Off (RO) still enabled (main output off), change input voltage to 264Vac, and record the fan output voltage:

Fan output = 0% / Standby output: 0%

Fan output = 0% / Standby output: 100%

Fan output = 100% / Standby output: 0%

Fan output = 100% / Standby output: 100%

| Model Level | Under-Voltage Protection (UVP) | OPEN | Unknown |
| --- | --- | --- | --- |

Specification / Procedure

Spec: Off @ 70Vac typ., Restore @ 80Vac typ.

Select any voltage model.

At light load (e.g. 10%), slowly reduce the input voltage until the unit shuts off. Record the voltage at which it turns off. Raise the input voltage slow and record the voltage at which it turns back on.

At full load, slowly reduce the input voltage until the unit shuts off. Record the voltage at which it turns off. Raise the input voltage slow and record the voltage at which it turns back on.

Check that there is no chatter and that the transitions are clean.

| Model Level | Over-Temperature Protection (OTP) | OPEN | Unknown |
| --- | --- | --- | --- |

| Model Level | Lightning Surge | OPEN | Unknown |
| --- | --- | --- | --- |

| Model Level | PFC Voltage Drop at 90Vac - overload | OPEN | Has Issue |
| --- | --- | --- | --- |

Specification / Procedure

Issue: At 90Vac, the PFC fails to maintain 395Vdc output at/near full load and above. Currently, it is understood that the SNSMAINS signal is faulty/noisy, causing TEA2376DT to erroneously reduce voltage loop gain, causing the PFC output voltage to fall out of regulation. There are protections indicated. The PFC "does not think anything is wrong", but the ship is sinking.

Mentioned here: https://github.com/GlobTek-Engineering/GTM965500P/issues/66#issuecomment-4164434018

Below: Output voltage (yellow), PG signal (blue), PFC voltage (magenta). Conditions: Approaching 145% load @ 90Vac input

[image: https://github.com/user-attachments/assets/74615967-e722-4f3c-8a99-d9e7e29c6439]

Improvement was gained by adding an RC filter to SNSMAINS which allowed operation at 100% load / 90Vac, but the issue was still not totally eradicated because the phenomenon again arises at overloads exceeding roughly 145%. Perhaps this is sufficiently beyond the specified range to safely ignore, but on the other hand, it's persistence still suggests a sensitivity issue that may rear its head in other ways later.